7000 Series Solenoid Proportional Valve

Acequip 7000 series metal-sealed solenoid proportional valves with exceptional sealing and corrosion resistance. These valves utilize high-purity metal sealing materials to minimize particulate matter and residual pollutants, reducing equipment failure rates and ensuring long-term efficiency. Leakage rate lower than 1*10-11Pa.m³/s. Offering both teflon-metal seal (70-) and all-metal seal (71-) specifications to meet different application requirements.

- Compliant with RoHS

- Optional Teflon metal seal (70-) and all metal seal (71-)

- Maximum flow range: 0-200 l/min

- Leakage rate: ＜1*10-11Pa.m³/s

7000 Series

型号选择Model Selection

70（71）45: 0-25 SLM

50: 0-43 SLM

55: 0-55 SLM

60: 0-115SLM

65: 0-150SLM

70: 0-200SLM

Model

Material

SS316: 316

Power

12: 12V

24: 24V

Connection

E4V: 1/4''VCR

E6V: 3/8" VCR

X: 定制型号

Max Flow

25  SLM

43  SLM

55  SLM

115  SLM

150  SLM

200  SLM

- Teflon型金属密封 型号示例： 7045-SS316-12-E4S-25SLM

- 全金属密封型号示例： 7145-SS316-12-E4S-25SLM

- 如需定制，X的具体数值请咨询销售代表。

Specifications

| MODEL |  | 7000 Series |  |  |  |  |  |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Size |  | Medium Flow |  |  |  |  | High Flow |  |  | Same Size |
| Model | Teflon Metal Seal | 7045 | 7050 | 7055 |  | 7060 |  | 7065 | 7070 | Air calibration at 0.1 MPa |
|  | All Metal Seal | 7145 | 7150 | 7155 |  | 7160 |  | 7165 | 7170 |  |
| Max Flow(SLM) |  | 0-25 | 0-43 | 0-55 |  | 0-115 |  | 0-150 | 0-200 |  |
| Operating differential pressure |  | 0-0.6MPa |  |  |  |  | 0-0.4MPa |  |  |  |
| Proof Pressure |  | 1MPa |  |  |  |  |  |  |  |  |
| Control | Control Voltage (Max) | 12V/24V可选 |  |  |  |  |  |  |  | DC直流 |
|  | Power Consumption | Max 2.5W |  |  |  |  |  |  |  |  |
| Filter |  | 800 mesh stainless steel (Inlet) (Filter for outlet on request) |  |  |  |  |  |  |  |  |
| Internal leak |  | ＜0.1 ml/min or less |  |  |  |  |  |  |  | within the proof pressure range of the gas |
| External Leak(He) |  | 1*10-11Pa.m³/s以下 |  |  |  |  |  |  |  |  |
| Operating temperature |  | 0℃-50℃ |  |  |  |  |  |  |  | The coil's copper resistance follows Rt = R0°C (1 + 0.004 × t°C). For voltage control, maintain a stable ambient temperature; otherwise, use current control. |
| Storage temperature |  | -5℃-70℃ |  |  |  |  |  |  |  |  |
| Wetted Material | Teflon Metal Seal | 70-: PTFE、SUS316、Electromagnetic stainless steel |  |  |  |  |  |  |  |  |
|  | All Metal Seal | 71-: PTFE、SUS316、Electromagnetic stainless steel |  |  |  |  |  |  |  |  |
| Connections |  | 1/4"VCR or 3/8"VCR |  |  |  |  |  |  |  | More options |
| Weight |  | About 523g |  |  | About 1356g |  |  |  |  | Fittings included |

7000 Series Medium Flow Solenoid Proportional Valve

7000 Series High Flow Solenoid Proportional Valve