Acequip Metal Seal（RS-485）MFC/MFM

A2-FCMRNCS/A2-FCMRVCS/A2-FMMRNS/A2-FMMRVS

- Product Description

- A high-performance metal-sealed digital (RS-485) mass flow controller, featuring a high-precision flow sensor that ensures an accuracy range of ±1% of set point (S.P.). With a metal-sealed design, it offers excellent corrosion resistance.

Features

- Flow range available from 2 sccm (milliliters per minute) to 50 slpm (liters per minute).

- Two types of control power supplies: Dual power supply (±15VDC) or wide voltage input (+12VDC to +32VDC).

- RS-485 digital communication interface, supporting both ASCII communication protocol and Modbus RTU protocol.

- Metal seal with Teflon valve seat.

- Leak rate: Max 1x10^-11 Pa·m³/sec (He).

- Configurable Options

- Multi-gas / multi-range configuration available.

- Configurable multifunctional LCD local display control interface.

Specifications

| Model | A2 |
| --- | --- |
| Full Scale Range(N2 eqv) | 2 SCCM |
| Response Time | ≤ 0.8 sec |
| Accuracy | ≥25% F.S. ：≤±1% S.P. |
|  | ＜25% F.S. ：≤± 0.25% F.S. |
| Linearity | ≤ ±0.5% of Full Scale |
| Repeatability | ±0.1% of Full Scale |
| Operating Differential Pressure | 2 SCCM – 10 SCCM：0.05 - 0.28 MPa |
|  | 6 SLM – 50 SLM：0.1 – 0.28 MPa |
| Leak Integrity | Max 1x10-11Pa.m3/sec (He) |
| Control Range | Single Gas： 2-100% of Full Scale |
|  | Multi Gas Multi Range：5-100% of Full Scale |
| Max operating Pressure | 0.48 MPa |
| Proof Pressure | 1 MPa |
| Operation Temperature | 5-45 Deg C |

| Control Valve | Normally closed/Normally Open Solenoid Valve |
| --- | --- |
| Fitting | 1/4"VCR or 1/4" Swagelok Compatible |
| Seal | All Metal Seal |
| Orientation | Free |
| Wetted Material | SUS316L, Ni, PTFE, Electromagnetic stainless steel |
| Surface Finish | TC，≤5RA |
| Weight | appr. 1 KG |

Mechanical

Electrical

| Analog Input Voltage | DC 0-5V |  |
| --- | --- | --- |
| Analog Output Voltage | DC 0-5V |  |
| Power Supply | ±15VDC | +15VDC ±2% 25mA |
|  |  | -15VDC ±2% 180mA |
|  | +12VDC-+32VDC | +12VDC 500mA - +32VDC 180mA |
| Power Consumption | ±15VDC | 3.1W |
|  | +12VDC- +32VDC | 6W |
| Analog Connector | 9Pin D-sub Male |  |
| RS-485 Connector | 6-Pin RJ-11 * 2 |  |

Dimensions

Acequip Metal Seal（RS-485）MFC/MFM

A2-FCMRNCS-E4S/A2-FCMRVCS-E4S/A2-FMMRNS-E4S/A2-FMMRVS-E4S (1/4”LOK)

Acequip Metal Seal（RS-485）MFC/MFM

A2-FCMRNCS-E4V/A2-FCMRVCS-E4V/A2-FMMRNS-E4V/A2-FMMRVS-E4V(1/4”M.VCR)

Acequip Metal Seal（RS-485）MFC/MFM

A2-FCMRNCS-IGW/A2-FCMRVCS-IGW/A2-FMMRNS-IGW/A2-FMMRVS-IGW (IGS 1.125"W-SEAL)

Part Number Identification

Acequip Metal Seal（RS-485）MFC/MFM

A2-FCMRNCS/A2-FCMRVCS/A2-FMMRNS/A2-FMMRVS